NOS mail docs -- G4AMJ/NQ0I and SM0RGV (Rev. 3)

Introduction

Intended audience

These documents are intended for the NOS system  operator  desiring to enable

the  forwarding  of mail to  other systems.  They are NOT intended as  a user

guide for the mail capabilities of NOS.

Background

This section of the NOS docs deals  with the intricacies  of mail forwarding.

You  should  read  and   understand  this  documentation   thoroughly  before

attempting  to  forward mail  through your  NOS box to  the AX.25  BBS world,

otherwise you might grossly  misconfigure  your  system  and  be  the unhappy

recipient of flames from BBS sysops.

This section does NOT deal  with the minutae  of the mailbox  and its various

commands;  it assumes that you understand concepts  such as user  areas (both

public  and private)  and how to  list and  send mail. If you need  help with

these, please look elsewhere in the NOS docs.

Apart  from  the usual domain.txt  and  other  files  necessary  for ordinary

functionality  of  NOS,  three files are  important  in  the  mail forwarding

process.  These  are :  /spool/forward.bbs,  /alias  and  /spool/rewrite. The

contents of these will now be addressed individually.

/spool/forward.bbs

This file describes the actions taken by  NOS in  forwarding  to AX.25 BBSes.

The file contains a series of forwarding records, each record being separated

by  a line  containing  two or  more  hyphens.  The template for a forwarding

record is:

BBS callsign

Connection route

Connection commands                <zero or more lines>

List of areas to be forwarded      <one per line>

------------                       <end of record>

BBS callsign

This  is  simply  the ordinary  call  of the  remote BBS.  A typical (but not

random!) entry might be simply the line:

sm0rgv

The callsign may be followed,  on the same line, by a comma separated list of

valid intervals when forwarding is  to take place.  Each valid  interval is a

four digit number:  the first two digits are the beginning hour of  the valid

interval,  the last two digits are the final hour of the  valid interval. For

example, if the first line of a forwarding record looks like:

sm0rgv 0006,1414

then forwarding to sm0rgv will take place only during hours  numbered 00, 01,

02,  03,  04,  05,  06 and 14. Ticks of the mbox timer outside of these times

will not cause mail  to  be  forwarded to  sm0rgv.  The  default interval for

forwarding is 0023.

Connection route

This  is  the method  by  which communication is  to be established  with the

remote BBS.  The first token on the line is the type of protocol  to be used.

This  is  one of  ax25,  netrom or  tcp.  Following this  is whatever further

information the chosen protocol requires  to make the connection.  An example

connection route for a simple ax25 connection on interface ax0 is:

ax25 ax0 g3dlh

Connection commands

Connection commands may,  optionally, follow the connection route. These take

the  form  of  a full  stop (period),  followed by the command which  will be

transmitted once the connection defined in the first line  of  the connection

route is established.

For example,  suppose that  we  wish  to  establish a netrom  connection with

sm0rgv-2,  through  the netrom  node  #sth67.  Then the connection  route and

connection command portion of the record would look like:

netrom #sth67

the beginning of the line; I have placed it here

indented by one column simply so that gateways

which handle this message do not complain at

having a line beginning with a full stop; this

convention is followed throughout this documentatione

If the station is reached through digipeating,  then the digipeater callsigns

should be in the ax25 route to the destination callsign. That is, if you wish

to forward traffic to w0ljf, using k2na as a digipeater, then you should have

the line:

ax25 route add w0ljf k2na

in your autoexec file.

List of areas to be forwarded

This is a list,  one per line,  of entries in the /spool/mail directory which

will be forwarded to the remote BBS. An entry of the form:

callsign

will  cause  the  file  /spool/mail/callsign.txt  to  be  scanned  for unread

messages.  Any such messages are sent to the remote BBS and deleted  from the

file.

One  can also  forward user  areas using this  mechanism.  To do this, simply

place a line containing the name of  the area  in the record.  So, to forward

amsat bulletins to the BBS, one would have a line:

amsat

This will  search  the  /spool/mail/amsat.txt  file;  any  messages contained

therein  which have  not been  forwarded  to  the  BBS  in  question  will be

forwarded.  They will NOT be deleted. The determining factor as to whether or

not  entries  are  deleted  is  that  if  the  filename  is  present  in  the

/spool/areas file, then there is NO deletion, otherwise there is.

Please note that ONLY  FILES IN  /spool/mail are checked.  In particular, the

outbound SMTP mail queue is NOT checked.

Changing the recipient address

Normally,  NOS uses the information in the  To:  header line to determine the

parameters used by the "S"  command during BBS forwarding.  As the To: header

is unchanged by all /alias and /spool/rewrite machinations,  the mail will be

sent to the BBS addressed precisely as  the originator  of  the message typed

it.  Occasionally,  one might want to change this behaviour.  In this case, a

line of the form:

area  new_address

in the list  of  areas to  be  forwarded will  replace  the  originally typed

destination with the string new_address instead.

/alias

The alias file is used to map LOCAL names to other names, which may be either

local or remote;  additionally,  from a single input message,  the alias file

permits one to produce multiple output messages

The format of a record in the alias file is very simple:

aliasname recipient1 recipient2 recipient3

<tab> or <SP> recipient4 ... recipientN

There is  no  separation  between records in  the /alias  file  other  than a

newline.

The  aliasname is  a local username;  that is,  it does  not  contain  an "@"

symbol.  When the alias file is processed,  if the destination of the message

matches precisely the aliasname,  then the mail  is redirected to  ALL of the

alieased recipients.

Scanning of the /alias file is performed by  the SMTP server.  The SMTP timer

(which controls  the SMTP  client)  is kicked  whenever  the mailbox  or SMTP

server queues something for delivery by SMTP.  Mail transport within a single

NOS system is performed through the SMTP client/server mechanism.  The result

of these facts is that as soon as a piece of mail is entered to  the mailbox,

the SMTP client is kicked and attempts to deliver the mail (which has already

been scanned by the rewrite mechanism \(em see below).  If the mail  is local

to the NOS system  (i.e.  no "@"  sign in the address),  then the /alias file

will be scanned and the name mappings take place.

A few lines in the /alias file might look something like:

bdale bdale@n3eua

local fred@k0yum bdale@n3eua bill@ai0c.co.usa.na

n5op@n5op jim@k0jtz n0esg@n0esg

g4bki g4bki@gb7bil._2712.gbr.eu

The system  must  know  how to  deliver traffic  to  each  of  the individual

addresses in the style in which they are entered  in the /alias file.  If the

system does not know how to deliver one of  the new addresses,  then  it will

send it to the SMTP gateway station defined by the 'smtp gateway' command.

Note that it is reasonable,  and sometimes desireable,  to have alias records

of the form:

area area dest1 dest2 ...

As the /alias file is scanned only once (see below),  this does not result in

an infinite recursion.

/spool/rewrite

The rewrite file is used to perform a one-to-one mapping  between destination

addresses as received  by NOS and  destination addresses as  actually used by

NOS.  Each record within the rewrite file comprises a single line, containing

either two or  three entries separated by  spaces.  The  first  field  is the

template field; if a destination address matches the template, it is replaced

by the second field. The third field, which is optional, is the single letter

"r",  which,  if present, tells NOS to rescan the rewrite file, using the new

destination address to attempt to match against the templates.

A template may contain asterisks.  These stand for a match of  any  number of

characters (including zero). In the second field, the character "$", followed

by a single digit in the range 1 to 9, represents the string that matched the

respective asterisk in the template. By way of example, suppose that there is

a line in the rewrite file which looks like:

*@* $1%$2@g1emm.ampr.org

Then, any traffic reaching the system through the mailbox or the SMTP server,

but  which is  supposed to go  to a remote system,  will be  redirected to go

through g1emm.ampr.org.  Suppose that a user logson,  and sends  a message to

n0gbe@nq0i.  Then the rewrite file attempts to match "n0gbe@nq0i" against the

entry  *@*.  It matches,  and assignes $1  the value n0gbe,  and $2 the value

nq0i.  The mail file as written to the disk will no longer be  to n0gbe@nq0i,

but, rather, to n0gbe%nq0i@g1emm.ampr.org. U

The nomenclature  station1%station2@station3  means the final  destination is

station1@station2,  and this  traffic is  to  be  routed  through the gateway

station3.e

As  soon  as  a template  match is  found,  the conversion  is  performed and

scanning  is stopped,  unless the third "r"  field is present,  in which case

scanning restarts from the top of the file.

N.B. It is a good idea to have a line of the form:

*@*.ampr.org $1@$2.ampr.org

at the beginning of your rewrite file. This will cause all amprnet traffic to

be caught early in the rewrite scan,  and no further scanning (and, hence, no

unexpected substitutions) will take place.

Scanning procedure

The two files which are used  to  determine the  disposition  of  traffic are

scanned under slightly different circumstances.  Note that neither the /alias

nor the /spool/rewrite scan makes any actual  changes to the  contents of the

traffic. In particular, the To: field remains exactly as it was first entered

into the system.

There are four  possible  entry routes  for traffic  into  the  system: SMTP,

through the mailbox by  a user,  through  the mailbox by  a BBS,  and  via an

external program (like BM)  or creation of the files manually. NOS determines

if a piece of traffic was entered into the system by  a BBS by  looking for a

BBS system  ID  (like the "UNET-H$e"  block issued  by  NOS)  on the incoming

connection prior to messages being uploaded.

Traffic received by SMTP server

1.  The rewrite file is scanned and any changes applied  (unless  the traffic

was recieved  through the local mailbox;  in  that  case,  this step does not

occur);

2.  If the traffic appears to be local then the alias file is scanned and any

changes or explosions applied.

3.  Any copies local to the system are delivered;  copies for remote delivery

are placed in the SMTP queue.

Traffic received by mailbox from user

1. The rewrite file is scanned and any changes applied;

2. The traffic is passed to the SMTP client.

Traffic received by mailbox from BBS

1. The rewrite file is scanned and any changes applied;

2. The traffic is passed to the SMTP client.

Traffic entered by external mechanism

1. No scanning occurs;

2. The traffic is passed to the SMTP client.

Headers

Appropriate RFC-822  headers  are  added  to  all  incoming  traffic. Traffic

entering through the mailbox recieves a full  complement  of RFC-822 headers;

traffic coming through the SMTP server has only a "Received:" header applied.

On forwarding to a BBS,  if an item of  traffic contains BBS  R: headers, the

RFC-822  header is converted to an appropriate R:  line at the  time that NOS

forwards the message. (This change only occurs for BBS forwarding; forwarding

by SMTP retains the RFC-822 headers.)

Bulletin Identifiers (BIDs)

The AX.25  BBS  system  has evolved a reasonably  efficient  way  of reducing

overhead when forwarding bulletins.  When a bulletin is originated  on a BBS,

it   is  given   a  unique   bulletin  identifier  (BID).   This  BID  should

(theoretically)  travel with the bulletin, and should never be changed during

the distribution  of  the bulletin.  Each system keeps track  of all received

BIDs. If a forwarding station wishes to forward a bulletin to a BBS, then the

receiving station checks  its  local  list  of  known  BIDs  and  informs the

transmitting station if it already posesses the bulletin in question. The NOS

mailbox conforms  to  this  protocol.  Received BIDs  are stored  in the file

/spool/history, and are encoded in the Message-ID: header line of the message

by  NOS.  Messages forwarded from areas listed  in the /areas file  will have

their BID (re)generated from  the Message-ID:  line.  Note  that ALL messages

from public areas are forwarded with  a BID,  whether or not  the message was

produced  with  the  "SB"  command.  Like  other  BBSes,  NOS  will  inform a

transmitting station not to transmit a bulletin if it is one that NOS already

has locally; likewise, it understands similar messages from other stations to

which it tries to forward.

Note  that  the BID mechanism is  not a part  of  the SMTP world.  If you are

forwarding  bulletins through  SMTP,  there  is  no  mechanism  by  which the

receiving station can reject the attempted delivery of a bulletin, even if it

already exists on the recipient system.  (Note that a possible  workaround is

to  deliver  bulletins  to  TCP/IP  stations  using  TCP   instead  of  SMTP.

Alternatively,  one could use NNTP,  as NNTP commands utilise the Message-ID:

line,  from which the BID is derived.)  The BID is preserved  no matter which

mechanism is used to deliver the bulletin.

Traffic in practice

Now,  the big question is, how does one set up these various files to perform

intelligent  manipulation  of mail?  A number of examples  follow. Note that,

often,  there is more than one way to accomplish an  objective. The following

are merely examples (and not necessarily the most  efficient  method possible

for any given case). The format used will be:

typed destination -> intended destination

followed   by  the   necessary   entries  in  the  alias   (/alias),  rewrite

(/spool/rewrite) and forwarding (/spool/forward.bbs) files.

Using familiar names \(em SMTP destination

bdale -> bdale@n3eua.ampr.org

alias:

bdale bdale@n3eua.ampr.org

rewrite:

forward:

Exploding local mail

sysops -> nq0i, n5op@n5op.ampr.org

alias:

sysops nq0i n5op@n5op@ampr.org

rewrite:

forward:

Using familiar names \(em BBS forwarding

g4bki -> g4bki@gb7bil._2712.gbr.eu, to be forwarded by ai0c

alias:

rewrite:

forward:

ai0c

ax25 ax1 ai0c

g4bki g4bki@gb7bil._2712.gbr.eu

ai0c

Handling incoming bulletins by subject

tcpip@* -> nq0i, tcpip, bdale@n3eua.ampr.org, ai0c@ai0c Ua BBSe

alias:

tcpip nq0i tcpip bdale@n3eua.ampr.org ai0c

rewrite:

tcpip@* tcpip

forward:

ai0c

ax25 ai0c

ai0c

Let's walk through the above example.  An incoming item comes in addressed to

TCPIP@ALLUS.  A scan is made through the rewrite file,  and a match is found.

The item is redirected to tcpip.  The alias file is scanned;  a total of four

copies of the item  exist after this,  three  in local areas  tcpip, nq0i and

ai0c,  and one on the SMTP queue (for bdale@n3eua.ampr.org). When the mailbox

timer next ticks,  the mail in the local ai0c area  will be forwarded  on the

ax1 interface to ai0c.

Routing based on Hierarchical addressing

Wyoming -> KE7VS (SMTP)

Nebraska -> AG0N (BBS over the NETROM, NETROM ID WNBBS)

Europe -> W0LJF (BBS over AX.25)

alias:

rewrite:

*.noam            $1.na r

*.us              $1.usa.na r

*.usa             $1.usa.na r

*.ne              $1.ne.usa.na r

*.wy              $1.wy.usa.na r

*@*.*.wy.usa.na   $1%$2.$3.wy.usa.na@ke7vs

*@*.wy.usa.na     $1%$2.wy.usa.na@ke7vs

*.ne.usa.na     ag0n

*.eu            w0ljf

forward:

ag0n

netrom ax0 wnbbs

ag0n

----------

w0ljf

ax25 ax1 w0ljf

w0ljf

----------

Why is the example rewrite file apparently so complicated?  This is to handle

poorly constructed hierarchical  addresses in  a reasonable way.  A full U.S.

hierarchical address has the form:  callsign@BBS. #localid.state.usa.na. Many

states have no #localid field.  In the example rewrite file  above, the first

three  lines convert non-standard,  but frequently used,  U.S. designators to

the  more  standard  format.  It is  common  for  users  not  to  use  a full

hierarchical address if  the destination is  relatively local.  For eample, a

user might easily  use only  .wy instead of  the  full  .wy.usa.na  if  he is

geographically close to  Wyoming.  The second  grouping of two  lines handles

this problem. Note the third, "r", field in all the entries so far.

The remainder of the file handles properly  formatted hierarchical addresses.

The two Wyoming entries handle the cases  with and without  a #localid field.

Differentiation between these cases is not necessary for BBS forwarding.

General bulletin handling

The details of bulletin handling will vary somewhat  from place  to place, as

there are several  distinct styles of  bulletin handling currently in  use in

the AX.25  BBS world.  In general, it is necessary to arrange one's system so

that it accepts bulletins from BBSes,  forwards them to one or more stations,

and also handles intelligently bulletins input by users into NOS.

Suppose that  we  sish  to  handle  bulletins @JUNK.  We are  to deposit them

locally in the junk area, and also forward to BBS g4bki. We also know that we

generally receive @JUNK bulletins from g4amj,  a local BBS which handles much

bulletin traffic.

alias:

rewrite:

*@junk   junk

forward:

g4bki

ax25 ax1 g4bki

g4bki

junk

----------

g4amj

ax25 ax1 g4amj

g4amj

junk

----------

All incoming @JUNK traffic is  written to the  junk area (which  should be an

explicit entry in the /spool/areas file). Each tick of the mailbox timer, NOS

scans the junk area for traffic not forwarded to g4bki or  g4amj and attempts

to deliver unforwarded bulletins.  Usually,  g4amj will respond  with a "Have

it"  message and the bulletin  will  not be  forwarded.  Any  bulletins @JUNK

deposited  locally by  users will  automatically be  sent  to  both g4bki and

g4amj.

Questions and Answers

Q. Under what circumstances does NOS request reverse forwarding from a BBS?

A. NOS requests a reverse forward after completing any forwards of its own to

the  BBS.  If no traffic was queued  for a given BBS,  then  no connection is

attempted, so no reverse forward request is issued.

Q. What kinds of message types does the NOS mbox support?

A.  Basically,  NOS supports all two letter commands starting with an "S". If

the mailbox has not received an SID banner (the "UNET-H$e")  from a connected

station,  then an SF command will send a followup to the address specified on

the command line.  The SR command will send a reply to  the  current message.

One can also issue the command "SR <number>", where <number> is the number of

the  message to  which   you   want   to   generate   a   reply.   All  other

variations  cause  an X-BBS-Msg-Type: header to be added to the message. When

a message with such a line is  forwarded to  a BBS,  it  is  sent  to the BBS

with the appropriate message type as the second letter in the "S"  command to

the BBS.

If  NOS has received  a valid SID,  then  ALL S commands  are handled  by the

X-BBS-Msg-Type: mechanism outlined above.

Logic map of the mailbox

============== AX.25 === NET/ROM === Ethernet === Loopback =================

]                   ]                   ]                   ]

]                   ]                   ]                   ]

+--------------+    +--------------+    +--------------+    +--------------+

]              ]    ]              ]    ]              ]    ]              ]

]   Mailbox    ]    ] SMTP client  ]    ] SMTP server  ]    ] BBS Forward  ]

]              ]    ]              ]    ]              ]    ]              ]

+--------------+    +--------------+    +--------------+    +--------------+

]                   ^                   ]                   ^

]                   ]                   ]                   ]

v                   ]                   v                   ]

+--------------+    +--------------+    +--------------+    +--------------+

]              ]    ]              ]    ]              ]    ]              ]

] Add RFC822   ]    ] Use MX or A  ]    ] Add Received ]    ] Add own R:   ]

] header suite ]    ] type records ]    ] line         ]  +>] line         ]

]              ]    ]              ]    ]              ]  ] ]              ]

+--------------+    +--------------+    +--------------+  ] +--------------+

]                   ^                   ]          ]        ^

]                   ]                   ]          ]        ]

v                   ]                   v          ]        ]

+--------------+    +--------------+    +--------------+  ] +--------------+

]              ]    ]              ]    ]              ]  ] ]              ]

] Get Rewrite  ]    ] Use optional ]    ] Apply Rewrite]  ] ] Strip RFC822 ]

] file address ]    ] SMTP gateway ]    ] file address ]  ] ] header suite ]

]              ]    ]              ]    ]              ]  ] ]              ]

+--------------+    +--------------+    +--------------+  ] +--------------+

]                   ^                   ]          ]        ^

]                   ]                   ]          ]        ] Yes

v                   ]                   v          ]        ]

+--------------+           ]            +--------------+  ] +--------------+

]              ]   No      ]            ]              ]  ] ]              ]

] Local addr?  ]-------+   ]            ] Alias file   ]  +-] Any R: lines?]

]              ]       ]   ]            ]              ] No ]              ]

+--------------+       ]   ]            +--------------+    +--------------+

]               ]   ]                ]  ]  ]                ^

] Yes           ]   ]                ]  ]  ]                ]

v               ]   ]                v  v  v                ]

+--------------+       v   ]            +--------------+    +--------------+

]              ]    +--------------+    ]              ]    ]              ]

] Apply Rewrite]    ]              ] No ] Local        ]Yes ] /spool/mail/ ]

] file address ]--->] SMTP queue   ]<---] address?     ]--->] directory    ]

]              ]    ]              ]    ]              ]    ]              ]

+--------------+    +--------------+    +--------------+    +--------------+