!!

!! Deutscher Text siehe scc_ger.doc

!!

!!

!! sccdrv-1.4a

!! ===========

!!

!!

!! changes to 1.3: The Z8530 SCC driver is now a real tty driver.

!!                 I did not like Guido's idea patching serial.c

!!		   The serial driver changed from patchlevel to

!!		   patchlevel... I hope the code of tty_io.c does

!!		   not behave like that...

!!

!! changes to 1.4: well, the 1.1.* kernel series changed the whole

!!                 tty-driver structure... I hope it does n....

!!

!!

!! (error) reports regarding this version (sccdrv14a.dl1bke)

!! to dl1bke  f i r s t  please.

!!

!!

!! vy 73 de Joerg DL1BKE @ DK0MWX.DEU.EU or dl1bke@db0pra.ampr.org (smtp)

!!                Internet: dl1bke@Melaten.IHF.RWTH-Aachen.DE

!!

!! BTW: I am not the author of the driver. See scc.c for details.

!! mni tnx to pe1chl and pe1nnz for their great work.

0. Installation of the package

==============================

Run SCC-Install. If one of the patches fails please consult

chapter 2.

1. Initialization and attachment of the channels

================================================

To use the driver, 2 steps must be performed:

1. Global initialization of the driver in the kernel

2. Attachment of each channel in the packet software

The global initialization is needed to reset all SCCs and to

install a common interrupt handler.  Also, the hardware addresses

of the chips are defined in this step.  In the second step, each

channel is set up for the intended use.

1.1. Initialization

===================

Initialization is performed by setting the defines in the file

"/linux/drivers/char/scc_config.h". You can change a number of parameters:

################################################################################################

# For OptoSCC card e.g:

#

/* SCC hardware parameters */

#define NCHIPS	2		/* number of chips */

#define IOBASE	0x150    	/* base of first chip */

#define SPACE	4		/* spacing between chips */

#define AOFF	2		/* offset to channel A ctrl */

#define BOFF	0		/* offset to channel B ctrl */

#define DOFF	1		/* offset from ctrl to data */

#define INTACK	0x168      	/* INTACK ioport (vectored) or 0 for no INTACK (polling) */

#define IVEC	9		/* interrupt vector number */

#define CLK	4915200		/* clock frequency */

#define PCLK	1		/* PCLK or RTxC for clock */

#define HWTYPE	HWSCC		/* selection of special hardware types (see list above) */

#define HWPARAM 0		/* extra parameter for special hardware */

/* set to '1' if your card has an fullduplex divider */

/*		Channel  A  B	      Chip	*/

/*			======      ========   	*/

int SCC_Fulldup[8] =	{1, 1,  /* ...one...	*/

1, 1,  /* ...two...	*/

1, 1,  /* ...three...	*/

1, 1}; /* ...four...	*/

/* set to '1' if your modem supports external clocking */

/*		Channel  A  B	      Chip	*/

/*			======      ========   	*/

int SCC_Extclk[8] =	{0, 0,	/* ...one...	*/

0, 0,  /* ...two...	*/

0, 0,  /* ...three...	*/

0, 0};	/* ...four...	*/

/* set to '1' if your modem uses NRZ instead of NRZI coding */

/*              Channel  A  B         Chip      */

/*                      ======      =========   */

int SCC_NRZ[8] =        {0, 0,  /* ...one...    */

0, 0,  /* ...two...    */

0, 0,  /* ...three...  */

0, 0}; /* ...four...   */

/* set to '1' if you have and want ESCC chip (8580/85180/85280) support */

/*		Channel  A  B	      Chip	*/

/*			======      ========   	*/

int SCC_Enhanced[8] =	{0, 0,	/* ...one...	*/

0, 0,  /* ...two...	*/

0, 0,  /* ...three...	*/

0, 0};	/* ...four...	*/

/* SCC driver parameters */

#define MAXSCC		4	/* maximal number of SCC chips supported */

#define BUFSIZE		1024	/* minimal pre-allocated memory per buffer */

#define TPS             25  	/* scc_timer ticks per second */

/*********** END OF CONFIGURATION PARAMETERS ********************************************/

################################################################################################

# For Baycom (U)SCC card e.g:

#

/* SCC hardware parameters */

#define NCHIPS	2		/* number of chips */

#define IOBASE	0x300    	/* base of first chip */

#define SPACE	2		/* spacing between chips */

#define AOFF	4		/* offset to channel A ctrl */

#define BOFF	5		/* offset to channel B ctrl */

#define DOFF	-4		/* offset from ctrl to data */

#define INTACK	0      		/* INTACK ioport or 0 for no INTACK */

#define IVEC	7		/* interrupt vector number */

#define CLK	4915200		/* clock frequency */

#define PCLK	1		/* PCLK or RTxC for clock */

#define HWTYPE	HWBAYCOM	/* selection of special hardware types (see list above) */

#define HWPARAM 0		/* extra parameter for special hardware */

/* set to one if your card has an fullduplex divider */

/*		Channel  A  B	      Chip	*/

/*			======      ========   	*/

int SCC_Fulldup[8] =	{0, 0,  /* ...one...	*/

0, 0,  /* ...two...	*/

0, 0,  /* ...three...	*/

0, 0}; /* ...four...	*/

/* set to one if your modem supports external clocking */

int SCC_Extclk[8] =	{0, 0,	/* ...one...	*/

1, 0,  /* ...two...	*/

0, 0,  /* ...three...	*/

0, 0};	/* ...four...	*/

/* set to '1' if your modem uses NRZ instead of NRZI coding */

/*              Channel  A  B         Chip      */

/*                      ======      =========   */

int SCC_NRZ[8] =        {0, 0,  /* ...one...    */

0, 1,  /* ...two...    */

0, 0,  /* ...three...  */

0, 0}; /* ...four...   */

/* SCC driver parameters */

#define BUFSIZE	1024		/* minimal pre-allocated memory per buffer */

#define MAXSCC		4	/* maximal number of SCC chips supported */

#define TPS             25  	/* scc_timer ticks per second */

After you changed a parameter you have to recompile a new kernel image file.

If you changed TPS please don't forget to recompile param.c!

The channel number ranges from 0 to (2 * nchips) - 1,

where NCHIPS is the number of chips.

The crystal clock is specified as 4.9152 MHz.  Other frequencies

can be used, and this parameter should be adjusted accordingly.

The number of receiver buffers is 1024. If you see a non-zero

number in the "Space" column in the SCCSTAT output (see below)

you can increase this number.  It specifies the size of the

buffers that are pre-allocated for the receiver buffers of

all channels attached.

You can define your scc type in HWTYPE:

SCC type                 value

---------------------------------

PA0HZP SCC card          HWSCC

EAGLE card               HWEAGLE

PC100 card               HWPC100

PRIMUS-PC (DG9BL) card   HWPRIMUS

DRSI PC*Packet card      HWDRSI

BayCom (U)SCC card       HWBAYCOM

1.2. Attach commands

====================

When the linux has startup, the SCC driver has been initialized,

you can attach the channels in your packet software. This is done

by open the scc devices by using the attach asy command.

The SCC-drivers emulates the scc devices as serial asy ports,

this means e.g. that the baudrate can be set in the attach command.

Example Wampes:

#############################################################################################

# Wampes device attach

# NOTE: Interfacename and the device must be the same!!

# Usage: attach asy 0 0 slip|vjslip|ax25ui|ax25i|nrs|kissui <label> 0 <mtu> <speed> [ip_addr]

#

attach asy 0 0 kissui sc1 256 256 1200    # Attach SCC channel 1 in 1200 baud

attach asy 0 0 kissui sc2 256 256 1200    # Attach SCC channel 2 in 1200 baud

attach asy 0 0 kissui sc3 256 256 38400   # Attach SCC channel 3 in 38400 baud

attach asy 0 0 kissui sc4 256 256 4800    # Attach SCC channel 4 in 4800 baud

#                ^

#                 for WAMPES 921229 use here: ax25

#

Example JNOS:

############################################

# JNOS device attach

#

#attach asy sc1 0 ax25 sc1 256 256 1200

#attach asy sc2 0 ax25 sc2 256 256 1200

#attach asy sc3 0 ax25 sc3 256 256 300

#attach asy sc4 0 ax25 sc4 256 256 4800

#

#

It allows AX.25 communication without a TNC.  Only a MODEM is

needed. The parameters have the same meaning as in KISS mode.

In fact, the AX.25 mode is emulating an extended KISS TNC, so

the same commands can be used to set the parameters of the

interface (see below).

To be able to run fullduplex using an SCC in AX.25 mode, an

external divider must be available, that divides the baudrate

generator clock available on the TRxC pin by 32, and puts the

resulting signal on the RTxC pint of the same channel of the SCC.

Such a divider is not necessary for normal CSMA packet radio

operation, but interrupt overhead is slightly reduced if you

still install it.

1.3. SCC Parameters:

====================

The setting of parameters of the emulated KISS TNC is done in the

same way in the SCC driver. You can change parameters by using

the command param in NET or NOS or use the program: param.

param sc4 <paramname> <decimal-|hexadecimal value>

You can change the following parameters:

name	    : value

-------------------

speed       : 1200

maxdefer    : 0

txdelay     : 36

persist     : 128

slot        : 16

tail        : 8

fulldup     : 0

wait        : 48

maxkey      : 7

min         : 3

idle        : 30

group	    : 0

txoff       : 0

The parameters have the following meaning:

txdelay:

The delay (in units of 10ms) after keying of the

transmitter, until the first byte is sent. This is usually

called "TXDELAY" in a TNC.  When 0 is specified, the driver

will just wait until the CTS signal is asserted. This

assumes the presence of a timer or other circuitry in the

MODEM and/or transmitter, that asserts CTS when the

transmitter is ready for data.

A normal value of this parameter is 30-36.

persist:

This is the probability that the transmitter will be keyed

when the channel is found to be free.  It is a value from 0

to 255, and the probability is (value+1)/256.  The value

should be somewhere near 50-60, and should be lowered when

the channel is used more heavily.

slot:

This is the time between samples of the channel. It is

expressed in units of 10ms.  About 200-300 ms (value 20-30)

seems to be a good value.

This parameter should never be 0.

tail:

The time the transmitter will remain keyed after the last

byte of a packet has been transferred to the SCC. This is

necessary because the CRC and a flag still have to leave the

SCC before the transmitter is keyed down. The value depends

on the baudrate selected.  A few character times should be

sufficient, e.g. 30ms at 1200 baud. (value 3)

The value of this parameter is in 10ms units.

full:

The full-duplex mode switch. This can be one of the folowing

values:

0:   The interface will operate in CSMA mode (the normal

half-duplex packet radio operation)

1:   Fullduplex mode, i.e. the transmitter will be keyed at

any time, without checking the received carrier.  It

will be unkeyed when there are no packets to be sent.

2:   Like 1, but the transmitter will remain keyed, also

when there are no packets to be sent.  Flags will be

sent in that case, until a timeout (parameter 10)

occurs.

wait:

The initial waittime before any transmit attempt, after the

frame has been queue for transmit.  This is the length of

the first slot in CSMA mode.  In fullduplex modes it can be

set to 1 for maximum performance, or to a higher value to

give NET a chance to combine several packets in a single

transmission.

The value of this parameter is in 10ms units. It should

never be set to 0.

maxkey:

The maximal time the transmitter will be keyed to send

packets, in seconds.  This can be useful on busy CSMA

channels, to avoid "getting a bad reputation" when you are

generating a lot of traffic.  After the specified time has

elapsed, no new frame will be started. Instead, the trans-

mitter will be switched off for a specified time (parameter

min), and then the selected algorithm for keyup will be

started again.

The value 0 will disable this feature, and allow infinite

transmission time.

min:

This is the time the transmitter will be switched off when

the maximum transmission time is exceeded.  This parameter

is in seconds, and should never be 0.

idle

This parameter specifies the maximum idle time in fullduplex

2 mode, in seconds.  When no frames have been sent for this

time, the transmitter will be keyed down.  A value of 0 will

disable this feature, i.e. the transmitter will be keyed

indefinitely.

In CSMA mode this parameter is used to calculate the maxdefer

value: maxdefer = TPS * idle / slottime

maxdefer

Whenever the DCD or peristance test fails, this variable

is decremented by one. When maxdefer reaches 0 the transmitter

will be keyed immediatly.

txoff:

When this parameter has the value 0, the transmission of packets

is enable. Otherwise it is disabled.

group:

It is possible to build special radio equipment to use more than

one frequency on the same bad, e.g. using several receivers and

only one transmitter that can be switched between frequencies.

Also, you can connect several radios that are active on the same

band.  In these cases, it is not possible, or not a good idea, to

transmit on more than one frequency.  The SCC driver provides a

method to lock transmitters on different interfaces, using the

"param <interface> group <x>" command.  This will only work when

you are using CSMA mode (parameter full = 0).

The number <x> must be 0 if you want no group restrictions, and

can be computed as follows to create restricted groups:

<x> is the sum of some OCTAL numbers:

200  This transmitter will only be keyed when all other

transmitters in the group are off.

100  This transmitter will only be keyed when the carrier

detect of all other interfaces in the group is off.

0xx  A byte that can be used to define different groups.

Interfaces are in the same group, when the logical AND

between their xx values is nonzero.

Examples:

When 2 interfaces use group 201, their transmitters will never be

keyed at the same time.

When 2 interfaces use group 101, the transmitters will only key

when both channels are clear at the same time.  When group 301,

the transmitters will not be keyed at the same time.

Don't forget to convert the octal numbers into decimal before

you set the parameter.

SCC Status:

===========

Once the SCC driver has been initialized, some statistic

information can be shown using the sccstat command. The output of

this command shows one line of information.

Status:

HDLC                     Z8530               Interrupts

--------------------------------------------------------------

Sent       :       5     Space   :     0     RxInts :   366465

Received   :    3610     RxOver  :     0     TxInts :       88

RxErrors   :    5351     TxOver  :     0     ExInts :    35855

KissErrors :       0                         SpInts :     5731

Tx State   :    idle

The info shown is:

Sent		- number of frames transmitted

Received	- number of frames received

RxErrors	- number of receive errors (CRC, ABORT)

KissErrors	- number of KISS errors (should be zero...)

Space		- number of times the receiver buffer pool was found empty

RxOver		- number of receiver overruns

TxOver		- number of transmitter underruns

RxInts		- number of receiver interrupts

TxInts		- number of transmitter interrupts

EpInts		- number of receiver special condition interrupts

SpInts		- number of external/status interrupts

Tx State	- status of the Tx interrupt handler

(idle/defer/keyup/keywait/active/flush/tail, see scc.c)

An overrun is abnormal. If lots of these occur, the product of

baudrate and number of interfaces is too high for the processing

power of you computer. If "Space" errors occur, specify a higher

number of buffers in the define BUFSIZE in the "scc.h" file.

2. Problems

===========

We are poking around in somebody else's code, so everything may change

from one patchlevel to another... If the patches fail, try the

following:

2.1 /linux/drivers/char/Makefile

================================

Add "scc.o" to the definition of OBJS and "scc.c" to SRCS

2.2 /linux/include/linux/tty_driver.h

==============================

add the following DEFINE:

#define TTY_DRIVER_TYPE_SCC 0x0005

2.3 /linux/drivers/char/tty_io.c

================================

in tty_init() add the line

kmem_start=scc_init(kmem_start);

just before "return kmem_start".

2.4 Other problems

==================

If you have tx-problems with your BayCom USCC card please check

the manufacturer of the 8530. SGS chips have a slightly

different timing. Try Zilog... I have no information if this

driver works with baudrates higher than 1200 baud.

The SCC driver is NOT responsible for a "tried to free NULL pointer"

kernel error right after a reboot... This may occur with kernel versions

up to (at least) 1.1.18. Remember: The 1.1.* series is for alpha tests

-- use at your own risk ;-)